COLLEGE OF COMPUTER STUDIES

Systems Integration and Architecture 1 (SIA1) — 3rd Year, 1st Trimester

FINAL PROJECT DOCUMENTATION

Completed Submission

Project Title: Smart-Farm: IOT-based Agriculture & Automated Irrigation

Group Number / Section: LFCA311M178

Group Members and Roles

| Name | Role (per Section 5 of Project Guide) |
| --- | --- |
| Nikko Lopez Clavite | Systems Architect / Project Lead |
| Albert Joseph P. Fatalla | Embedded / Edge Developer |
| Klay Conrad Barbo | Middleware / Backend Developer |
| Faith Villapa | Security & QA Engineer |
| Jena Nicole Delos Santos | Frontend & Documentation Lead |

Course Facilitator: Francisco Ocsan

Date Submitted: 29 October 2026 (prepared date; update if the actual submission date differs)

Repository / Source Code Link:

Live Demo / Video Link:

# . Table of Contents

Contents are listed below. Refresh page numbers in Word after final layout, if required.

I.    Project Overview

II.   Stakeholder Analysis

III.  System Architecture (Viewpoints, Views, Perspectives)

IV.   Architecture Diagrams

V.    API Specification

VI.   Architecture Decision Records (ADRs)

VII.  ETL / Data Pipeline Description

VIII. Security Implementation

IX.   Testing and Validation Report

X.    Individual Contribution Summary

XI.   Appendices

# I. Project Overview

## 1.1 Problem Statement

Small-scale growers may not have continuous visibility into soil conditions across separate plots, so watering can be delayed or applied inconsistently. Manual checks require time and may miss changes between inspections. This project addresses that problem with two soil-moisture sensing points, live dashboard monitoring, and rule-based pump control. It provides threshold automation and manual control while recording readings for review.

## 1.2 Project Description

An ESP32 reads soil moisture for Plot A and Plot B and temperature/relative humidity from a DHT11. It converts the soil ADC readings to percentages, applies automatic irrigation thresholds, and publishes JSON telemetry over MQTT. A Node.js/Express middleware service receives telemetry, exposes REST endpoints, and writes event and periodic readings to Supabase. The browser dashboard polls the live API, displays sensor and pump status, charts trends/reports, and sends authenticated manual pump commands through the API and MQTT.

## 1.3 Objectives

- General Objective: Design and implement an IoT smart-farm prototype that monitors two plots and automates irrigation with a web dashboard and persistent telemetry.

- Specific Objective 1 (CLO1): Document the system architecture, components, data flows, deployment, and key quality considerations.

- Specific Objective 2 (CLO2): Integrate ESP32 telemetry and pump commands through MQTT and a Node.js/Express REST middleware API.

- Specific Objective 3 (CLO3): Apply authenticated, role-controlled account and management functions, and identify security and scaling requirements for deployment.

- Specific Objective 4 (CLO4): Define and conduct performance, interoperability, and failure-validation tests for the integrated system.

## 1.4 Scope and Limitations

Scope: one ESP32 node, two soil-moisture plots, one DHT11, one relay-controlled pump, local MQTT messaging, an Express API, Supabase authentication/database/storage, and a browser dashboard with live/history views, monthly reports, crop suggestions, notifications, profile management, and manager user controls. Automatic mode starts irrigation when both plots are below 35% and stops when both are at least 60%; manual ON/OFF and AUTO commands are also supported. Limitations: this is a prototype, not a validated commercial irrigation controller; sensor percentages depend on per-sensor calibration; only one node is configured; no flow/pressure, tank-level, leak, weather, or pump-current feedback is implemented; crop suggestions are based on stored temperature and fixed crop ranges; the supplied workspace contains no measured test report, live demo URL, or circuit drawing. Internet/cloud outages and local-network/MQTT configuration affect telemetry and remote commands.

# II. Stakeholder Analysis

The stakeholders below are inferred from the implemented features and the project roles.

| Stakeholder | Concerns / Interests | How the System Addresses It |
| --- | --- | --- |
| Farmers / operators | Know soil conditions and control watering without constant manual inspection. | Dashboard shows Plot A/B moisture, temperature, humidity, pump state, and trends; manual ON/OFF controls and automatic threshold mode are available. |
| Farm owner / administrator | Approve accounts, manage roles/users, receive pump/account notices, and review reports. | Manager-only routes enforce approved roles; notifications, user administration, monthly reports, and telemetry summaries are provided. |
| Project / operations team | Maintain reliable sensor, MQTT, API, and database integration; diagnose failures. | ESP32 serial logging, MQTT telemetry/control topics, REST endpoints, periodic persistence, and device-status endpoint support monitoring. |
| Farmer | Choose crops suited to observed conditions and avoid overwatering. | Crop suggestions use the stored recent temperature average and fixed crop temperature ranges; this is advisory, not a full agronomic model. |
| Account holders | Protect accounts and personal profile data; recover access. | Supabase Auth, approval checks, roles, hashed recovery answers, limited recovery attempts, and profile/photo validation are implemented. |

# III. System Architecture (Viewpoints, Views, Perspectives)

## 3.1 Architecture Overview

The edge tier samples sensors and controls the relay on the ESP32. MQTT carries telemetry to the middleware and carries dashboard-originated ON/OFF/AUTO commands back to the controller. The Node.js/Express tier subscribes to telemetry, maintains the latest in-memory reading, exposes REST APIs, and writes event and scheduled logs to Supabase. The presentation tier is the browser dashboard: it polls the live API, falls back to saved history when telemetry is stale, and uses Supabase Auth sessions for protected API actions. Soil readings and automation are evaluated on a nominal three-second interval; telemetry is published on pump-state changes or every 15 seconds. The dashboard polls live telemetry every 2.5 seconds.

## 3.2 Views

| View Type | Description of This System's View |
| --- | --- |
| Functional View | ESP32 sensing and threshold control; MQTT telemetry and commands; Express API; Supabase auth/storage/database; dashboard, reports, notifications, profile, and user-management flows. |
| Information View | Telemetry JSON: temperature, humidity, soilMoistureA, soilMoistureB, pumpState, trigger, and optional initiatedByUserId. Persisted sensor_logs fields include temperature, humidity, soil_moisture_a/b, pump_state, trigger_type, initiated_by_user_id, and created_at. Related data includes profiles, account_recovery, notifications, notification_reads, and profile photos. |
| Deployment View | Prototype deployment: ESP32 and sensors/relay on a Wi-Fi LAN; MQTT broker reachable on the LAN; Node.js/Express service; Supabase cloud Auth/DB/Storage; browser dashboard served as static files by Express. Firmware and backend currently contain deployment-specific host/configuration values. |
| Concurrency View | ESP32 runs a main loop, reads sensors about every 3 s, runs local auto-control, services MQTT, and publishes on events or each 15 s. MQTT callbacks handle commands. Express serves concurrent HTTP requests; an in-memory latest-reading cache is shared, with a 5-minute timer saving periodic readings. Browser polls live telemetry every 2.5 s. |

## 3.3 Perspectives (Quality Properties)

| Perspective | How It Is Addressed |
| --- | --- |
| Security | Supabase Auth access tokens are validated by the API; approved-account and manager checks protect routes. Sign-up approval, role restrictions, recovery answer hashing (bcrypt via pgcrypto), input bounds, image signature/size checks, and recovery throttling are present. Limitations: HTTP and MQTT port 1883 are not TLS in the shown configuration; CORS is permissive; Wi-Fi credentials are embedded in firmware source and must be moved to protected configuration before sharing/deployment. |
| Performance | The edge evaluates sensors nominally every 3 s, publishes periodic MQTT telemetry every 15 s (or immediately on state change), and the dashboard polls the live API every 2.5 s. The backend pages large historical/monthly queries in batches. No benchmark has been run; performance targets must be validated on the deployment hardware and network. |
| Availability / Reliability | The dashboard detects stale live telemetry after 30 s and falls back to saved history. Event changes and 5-minute periodic logging support observability. However, Wi-Fi setup and MQTT reconnect use blocking loops, so disconnected broker/network conditions can pause sensing/control; the DHT failure path skips control evaluation for that cycle. No redundant controller, broker, network, or power supply is configured. |

# IV. Architecture Diagrams

[Insert your system architecture diagram(s) here — e.g., tier/component diagram, sequence/data-flow diagram, deployment diagram (UML or equivalent). Label each figure (Figure 1, Figure 2, ...) and reference them in Section III.]

Figure 1. [Diagram Title]

Figure 2. [Diagram Title]

# V. API Specification

The consolidated table lists the Express routes implemented in smartfarm-backend/server.js. ?Bearer JWT? means a Supabase Auth access token validated by the middleware; manager routes additionally require an approved farm_owner or administrator profile. Public routes are unauthenticated. Error responses use JSON {"error":"..."} unless a 204 response is specified.

| Endpoint | Authentication / Request | Purpose / Response / Errors |
| --- | --- | --- |
| GET /api/telemetry/live | Public; no parameters. | Returns the latest in-memory telemetry object, including timestamp; initial values are zero with timestamp null. |
| GET /api/telemetry/history | Public; optional query limit (default 50). | Returns newest sensor_logs rows with id, measurements, pump_state, trigger_type, created_at. Database failure: 500 JSON error. |
| GET /api/telemetry/crop-insights | Public; no parameters. | Returns average_temperature, reading_count, period_start, period_end for approximately the prior three months. Database failure: 500 JSON error. |
| GET /api/telemetry/reports/months | Bearer JWT; approved farm_owner/administrator. | Returns {"months":["YYYY-MM", "... "]}. Database/read errors: 500 JSON error. |
| GET /api/telemetry/reports/monthly | Bearer JWT; approved farm_owner/administrator; query month=YYYY-MM. | Returns monthly labels, daily soilMoisture/temperature/humidity averages, overall averages, and readingCount. Invalid month: 400; database failure: 500. |
| POST /api/pump/control | Bearer JWT; approved account; JSON {"command":"ON¦OFF¦AUTO"}. | Publishes MQTT control command and authenticated user id; success returns status/message. Invalid command: 400; publish failure: 500. |
| GET /api/config | Public; no parameters. | Returns Supabase project URL and browser anon key for client initialization; does not return service-role key. |
| POST /api/auth/recovery/question | Public; JSON {"email":"..."}; maximum 5 attempts/IP per 15 minutes. | Returns {questionId} or null; service failure: 503; throttled: 429. |
| POST /api/auth/recovery | Public; JSON {"email":"...","questionId":"...","answer":"..."}; same throttle. | Validates recovery data when enrolled, then requests Supabase recovery email; returns a generic response to reduce account enumeration. Service failure: 503; throttled: 429. |
| GET /api/profile | Bearer JWT; signed-in user. | Returns current profile, email, and enrolled recovery question id. Missing/invalid token: 401; missing profile: 403. |
| PATCH /api/profile | Bearer JWT; approved account; JSON full_name; optional recovery and photo data URL. | Updates own name, recovery answer, and/or profile photo. Name 1?120 chars; answer 2?120 chars; JPEG/PNG/WebP data URL up to 2 MB with signature validation. Invalid input: 400; storage/database errors: 500/502. |
| GET /api/users | Bearer JWT; approved farm_owner/administrator. | Returns user profiles with email for manager user administration. Authorization errors: 401/403; service/database errors: 500. |
| PATCH /api/users/:id | Bearer JWT; approved farm_owner/administrator; optional full_name, role, is_approved. | Updates permitted profile fields; only farm_owner changes roles; self-change and last-manager protections apply. Invalid input: 400; forbidden: 403; missing: 404; conflict: 409. |
| DELETE /api/users/:id | Bearer JWT; approved farm_owner/administrator. | Deletes another permitted user, profile, and profile photo; cannot delete self or last manager. Responses: 204 on success, otherwise 400/403/404/409/500. |
| GET /api/notifications | Bearer JWT; approved account. | Returns up to 30 audience-appropriate notifications with per-user isRead and unreadCount. Errors: 401/403/500. |
| POST /api/notifications/:id/read | Bearer JWT; approved account. | Marks an eligible notification read; returns 204. Missing/ineligible notification: 404; errors: 401/403/500. |
| GET /api/devices | Public; no parameters. | Returns device metadata and an online indication derived from telemetry age. Note: some returned device metadata is static and should be reconciled with actual hardware before relying on it. |

The table below is consolidated; request bodies and representative response shapes are summarized per endpoint.

# VI. Architecture Decision Records (ADRs)

Three decisions below are inferred from the implemented code. Status is marked Accepted (implemented); confirm these decisions with the project team.

| ADR No. / Title | ADR-01: Use MQTT between edge, middleware, and control clients |
| --- | --- |
| Status | Accepted (implemented) |
| Context | ESP32 telemetry and pump commands need a lightweight transport on the local network, while the browser should not connect directly to the hardware. |
| Decision | Use PubSubClient on the ESP32 and MQTT topics smartfarm/telemetry and smartfarm/pump/control; Node.js subscribes to telemetry and publishes control commands. |
| Alternatives Considered | Direct HTTP from ESP32 to server; browser-to-device direct connection; polling the ESP32. |
| Consequences / Trade-offs | Decouples edge and web tiers and supports event delivery. Requires a reachable broker and topic/network configuration. Shown firmware uses non-TLS MQTT; production needs authenticated, encrypted broker access. |
| ADR No. / Title | ADR-02: Keep threshold control at the ESP32 edge |
| Status | Accepted (implemented) |
| Context | Pump safety and timely watering should not depend solely on a browser request. |
| Decision | Evaluate soil percentages locally about every 3 seconds; in AUTO, start when both plots are below 35% and stop when both are at least 60%. Relay is active-low. Manual commands select ON/OFF or return to AUTO. |
| Alternatives Considered | Cloud-only threshold decisions; separate independent thresholds per plot; fixed-duration cloud pump commands. |
| Consequences / Trade-offs | Local evaluation reduces network dependence during normal connected operation and hysteresis thresholds reduce rapid toggling. The current loop can block during Wi-Fi/MQTT reconnect and skips evaluation when DHT reads fail; behavior needs robust failure handling and hardware tests. |
| ADR No. / Title | ADR-03: Use Supabase Auth and role-gated middleware with persistent telemetry |
| Status | Accepted (implemented) |
| Context | The dashboard requires user sessions, approval/role management, recovery, and a durable history for reports. |
| Decision | Use Supabase Auth for accounts, verify bearer tokens in Express, gate approved and manager routes by profile role, and store event/periodic readings in Supabase. Use Supabase storage for profile photos. |
| Alternatives Considered | Custom password/session store; local-only telemetry; direct browser access to privileged database operations. |
| Consequences / Trade-offs | Delegates identity and persistence and supports reporting/admin flows. Depends on cloud connectivity and correct Supabase policies/configuration; project must validate RLS, TLS, backups, service-role secret handling, and operational recovery. |

All three ADRs are presented in the table below.

# VII. ETL / Data Pipeline Description

| Extract | What raw data is captured, from which sensor(s), at what interval. |
| --- | --- |
|  | ESP32 reads two analog soil sensors (GPIO34 Plot A, GPIO35 Plot B) and a DHT11 (GPIO23) about every 3 seconds. Soil raw ADC values are converted to moisture percentages using dry/wet calibration constants. |
| Transform | Validation rules, unit conversions, filtering/outlier removal applied. |
|  | Arduino map() converts the ADC range to 0%-100% and constrain() clips the result to that range. Telemetry is serialized as JSON. Backend parses JSON, timestamps it, and maps camelCase fields to snake_case database columns. Monthly reports aggregate daily/overall averages. No smoothing or outlier rejection is implemented; current calibration constants are prototype values and require field calibration. |
| Load | Where and how clean data is stored (database/table schema). |
|  | ESP32 publishes smartfarm/telemetry on pump-state changes and every 15 seconds. Express caches the latest sample in memory, immediately inserts EVENT_AUTO/EVENT_MANUAL records into Supabase sensor_logs, and saves the latest reading every 5 minutes as PERIODIC. Logs include temperature, humidity, both soil percentages, pump state, trigger type, optional manual initiator user id, and created_at. |

# VIII. Security Implementation

- Authentication mechanism used: Supabase Auth email/password sign-in. The browser sends the Supabase access token as a Bearer token; Express validates it through Supabase Auth and applies account-approval and role checks. Supabase service credentials remain server-side in environment configuration.

- Input validation applied on: pump command allowlist (ON/OFF/AUTO); monthly report month format (YYYY-MM); profile full name length (1-40); recovery question/answer ids and length (2-40); profile photo JPEG/PNG/WebP data URL, signature, and 2 MB maximum; user role and approval types; and recovery request throttling.

- Data-in-transit protection: The supplied configuration uses HTTP for the local web/API service and MQTT on port 1883. Treat this as a prototype-only limitation. Deploy HTTPS and MQTT over TLS with broker authentication before use outside a trusted isolated network.

- Other safeguards: approved-account and role-based access control; farm-owner-only role changes; prevention of self-deletion/self-approval changes and removal of the last owner/administrator; bcrypt-hashed recovery answers using pgcrypto; generic recovery responses and a limit of five recovery attempts per IP per 15-minute window; image signature/size checks; and server-side validation.

# IX. Testing and Validation Report

The scenarios below are a validation plan, not completed test evidence. No automated test files or measured results were present in the supplied workspace. Run each test on the target hardware/network and replace ?Not executed? with actual measurements before submission.

| Test Type | Scenario / Method | Expected Result | Actual Result / Analysis |
| --- | --- | --- | --- |
| Performance | Planned: on the target LAN, poll GET /api/telemetry/live from 20 concurrent clients at the dashboard polling cadence for 10 minutes; record p50/p95 latency, errors, and API CPU/memory. | Proposed target: p95 response under 1 s with no server errors under this small prototype load; sensor/control loop remains responsive. | Not executed. No benchmark harness or results were supplied; measure on target deployment and report the observed percentiles and error count. |
| Interoperability | Planned end-to-end test: ESP32 publishes a valid telemetry JSON message to MQTT; verify middleware cache, Supabase event/periodic row, dashboard display; send authenticated ON/OFF/AUTO command and verify MQTT receipt and relay state. | Telemetry fields and values propagate with correct names/units; command is accepted only for an approved authenticated user and relay behavior matches command/auto mode. | Not executed on hardware in the supplied workspace. Firmware, broker, Supabase, and credentials were not available for a confirmed integration run. |
| Reliability / Failure | Planned: disconnect MQTT broker/network and induce a DHT read failure; observe pump state, serial output, dashboard stale status, reconnection, and recovery of telemetry after service restoration. | Dashboard flags telemetry stale after 30 s; service recovers after reconnect without a false successful reading; pump fails to a documented safe state and resumes automation safely. | Not executed. Code inspection shows blocking MQTT reconnection and that DHT failure returns before automation evaluation; test and remediate these cases before any unattended deployment. |

## 9.1 Summary of Findings

No test execution results were supplied, so no passing results or performance measurements are claimed. Source inspection confirms the nominal sensor-processing interval (3 s), MQTT periodic publish interval (15 s), dashboard live polling interval (2.5 s), and stale-reading threshold (30 s). The firmware uses blocking Wi-Fi/MQTT reconnection loops; when MQTT is disconnected, sensor automation can pause while reconnecting. The DHT read failure path also skips the remainder of that sensor cycle, including the irrigation evaluation. Test these behaviors on hardware and address them before safety-critical use. The contribution table is role-aligned with the supplied project roster and observed code; members should verify individual attribution.

# X. Individual Contribution Summary

| Name | Role | Specific Contributions |
| --- | --- | --- |
| Nikko Lopez Clavite | Systems Architect / Project Lead |  |
| Albert Joseph P. Fatalla | Embedded / Edge Developer |  |
| Klay Conrad Barbo | Middleware / Backend Developer |  |
| Faith Villapa | Security & QA Engineer |  |
| Jena Nicole Delos Santos | Frontend & Documentation Lead |  |

# XI. Appendices

- Appendix A — Full Source Code Listing or Repository Link

- Appendix B — Circuit/Wiring Diagram of the Edge Node(s)

- Appendix C — Full Sensor Datasheet References

- Appendix D — Raw Test Data / Logs

- Appendix E — Screenshots of Dashboard and API Responses